VoiceOverLoRa (VLoRa)

LoRa Radio Link Statistics & Bandwidth Analysis

Built on the Nucleus platform — Natak Mesh (natakmesh.com)

# 1. LoRa Radio Configuration

Statistics below reflect the production VLoRa deployment on two Nucleus nodes (Alpha / Bravo), measured and calculated during live testing.

| Parameter | Value |
| --- | --- |
| Radio hardware | RAK4631 (SX1262) |
| Frequency | 915 MHz |
| TX Power | 30 dBm |
| Modem Preset | 6 (SHORT_TURBO family) |
| Bandwidth | 250 kHz |
| Spreading Factor — Original | SF11 |
| Spreading Factor — Production (voice-viable) | SF7 |
| Coding Rate | 4/5 (CR=1) |

# 2. Spreading Factor Airtime Comparison

Airtime per 72-byte LoRa packet (the VLoRa voice packet size — 9 × Codec2 3200bps frames) across spreading factors at fixed 250kHz bandwidth. This is the core finding that determined the project's final radio configuration.

| Spreading Factor | Airtime/Packet | 4s Speech = Packets | Total Airtime (4s clip) | Real-Time Viable? |
| --- | --- | --- | --- | --- |
| SF7 | 64.1 ms | ~23 | 1.5 s | Yes — faster than real-time |
| SF8 | 112.9 ms | ~23 | 2.6 s | Yes — near real-time |
| SF9 | 205.3 ms | ~23 | 4.7 s | Marginal — slight lag |
| SF10 | 369.7 ms | ~23 | 8.5 s | No — noticeable delay |
| SF11 (original) | 821.2 ms | ~23 | 18.9 s | No — not viable for voice |

Production deployment uses SF7. At the original SF11 configuration, a 4-second voice transmission required up to ~20 seconds of radio airtime to fully clear, causing severe packet loss as data queued faster than the radio could transmit it. SF7 reduced per-packet airtime by roughly 13×, resolving both the latency and packet-loss issues confirmed in field testing.

# 3. Measured Field Performance

Real values captured from running systems during live bidirectional voice testing between Alpha and Bravo nodes.

| Metric | Measured Value |
| --- | --- |
| SNR range during testing | 10.5 – 14.0 dB |
| Typical SNR (steady state) | ~12.5 – 13.5 dB |
| Codec2 mode | 3200 bps |
| Codec2 frame size | 8 bytes (20 ms audio/frame) |
| LoRa voice packet size | 72 bytes (9 frames = 180 ms audio) |
| Bad/corrupted frames (post-fix, SF7) | 0 — confirmed across multiple transmissions |
| Frame decode accuracy (post-fix) | 100% (frames decoded = RTP frames injected) |
| Perceived end-to-end latency (SF7, streaming decode) | ~1–2 seconds |
| Packet loss (SF11, pre-fix, Bravo→Alpha) | ~21% |
| Packet loss (SF11, pre-fix, Alpha→Bravo) | ~14% |

# 4. Link Budget Summary (SF7 / BW250)

| Traffic Type | Approx. Airtime Share | Active Condition |
| --- | --- | --- |
| VLoRa Voice (Codec2 3200bps) | 1.5s per 4s clip transmitted | PTT active |
| CoT / SA / Discovery / Chat | Single small packets, ~10–40s intervals | Continuous, low duty cycle |

# Notes

- All spreading factor airtime figures calculated using the standard LoRa airtime formula (Semtech AN1200.13), parameterized for SF/BW/CR/payload as configured on the deployed RAK4631 hardware.

- Field-measured values (SNR, packet loss, frame accuracy) captured from live systemd journal logs during bidirectional testing between Alpha (10.20.24.1) and Bravo (10.20.25.1) nodes.